Plot the slides displacement, xB, and the required crank input torque, Tin with the crank input angle α

| Flexible Segment Length L | 5 |
| --- | --- |
| Crank Length r2 | 3 |
| Offset e | 0.5 |

Assumptions:

- The slider always remains in contact with the ground

- No frictional resistance to slider motion

- Flexible segment is initially straight

- Slider does not support horizontal force, so the force on the end of the flexible coupler is vertical (n=0).

Derivation

For the compliant beam, the coordinates of the tip are approximated as:

x=l1-γ1-cosΘ

y=γlsinΘ

For a transverse end force, γ≈0.85.

Coupler length r3=γl

Fixed Link Offset r5=1-γl

The PRB model is a standard four-bar slider-crank mechanism. We define the position of the slider xB by summing the horizontal components of the vector loop:

xB=r2cosϕ+r3cosϕp+r5

Where r2 is the crank length, ϕ is the input crank angle, and ϕp is the pseudo-rigid body angle (the angle of the rigid coupler r3).

The vertical constraint, assuming the slider moves along the x-axis with an offset e, is:

r2sinϕ-r3sinϕp=c

Where c is the vertical distance from the origin to the slider path.

The compliance of the flexible couples is simulated by a torsional spring placed at the virtual pivot (point A). The spring constant K is derived from the beam’s material and geometric properties:

K=γKΘEIl

Where E is Young’s modulus, I is the moment of inertia, and KΘ is the stiffness coefficient (standard value for this loading is ≈2.25).